Request for Information #15-2026

Offshore Terminal Tugs – New Build

Distributed on: September 17, 2026

Response Requested by: October 19, 2026

ExxonMobil Guyana Limited (EMGL)

- Objective

This Request for Information (RFI) seeks detailed technical information from suppliers for meeting EMGL's bid selection requirements for up to six (6) New Build Offshore Terminal Tugs for a firm charter period in support of the EMGL’s offshore oil production program for multiple floating production, storage and offloading units (FPSO) in and around Georgetown, Guyana.

This RFI has several objectives:

First, we may use responses from you and the other participants to validate and further develop our team’s recommendations for inclusion in the Request for Proposal (RFP) for Offshore Terminal Tugs in and around Georgetown, Guyana.  The RFI process allows us to seek your input and develop a list of qualified bidders before structured bid rules take effect. If a supplier is exploring potential partnership options for qualification to perform a scope of work, it is not mandatory that a formal legal partnership has been established. Any technically qualified business model may be considered for inclusion in the RFP process. If this applies to you, clearly describe and explain the potential partnership intent in your response.

Second, the RFI process provides you with an opportunity to begin developing your responses to some of the potential requirements if you are selected for the RFP.

Suppliers should note that the anticipated RFP will be issued with a firm two (2) week response window. Given the accelerated procurement schedule, no extensions to the RFP submission deadline will be considered. Suppliers are therefore strongly encouraged to continue all prerequisite planning, market engagement, technical evaluations, and pricing activities during the RFI phase to ensure the timely submission of a fully developed proposal, including pricing for both CAPEX and OPEX requirements.

Finally, EMGL in its sole discretion, may divide or reallocate the services described in this RFI and in a future RFP (Request for Proposal) into several enabling agreements that may be awarded to more than one bidder.

Please be aware that this RFI does not imply any commitment at this stage from EMGL to acquire goods or services from any supplier. All expenses or costs incurred by or for your firm as a result of your participation in the RFI and any subsequent RFP process shall be at your sole cost and expense. Your participation in the RFI process does not guarantee an invitation to participate in an RFP process. There may be no RFP or Agreement issued related to this RFI.

- Background

General

ExxonMobil Guyana Limited (EMGL) plans to put in place a goods and services agreement for provision of up to six (6) New Build Terminal Offshore Tugs in and around the Georgetown area.

The scope of this Request for Information (RFI) is to identify supplier(s) who can provide Terminal Offshore Tugs to EMGL as follows:

- Charter Particulars

- Charter Type:	Time charter

- Charter Duration:	Firm charter period for each Vessel upto ten (10) years; each Charter shall include Charterer's options to extend the charter duration in one-year increments up to twenty (20) years total charter period.

- Delivery Area:	Georgetown, Guyana

- Commencement:	Charterer’s desired Charter commencement dates are as follows:

Vessel 1	3Q 2029

Vessel 2:	4Q 2029

Vessel 3:	1Q 2030

Vessel 4:	2Q 2030

Vessel 5: 	3Q 2030

Vessel 6:   4Q 2030

Owners are invited to propose their own charter commencement dates to reflect e.g. shipyard production schedules if newbuild vessels are to be offered.

- Trading Range:	Guyana coastal waters and exclusive economic zone (EEZ), as well as surrounding areas of the Western Atlantic and other navigable ports as Charterer may designate.  Company reserves the right to change charter area when exercising options and/or extensions.

- Compliance:	Vessels and Owner must be, and remain, in full compliance with all applicable international conventions, laws, regulations, and/or other requirements of the country of the Vessels’ registry, and of the countries, ports and/or places to which the Vessel may be ordered while in Charterer’s service.  For purposes of this solicitation, it may be assumed that there are no limitations (except those sanctioned by US Government) regarding the country of the Vessels’ registry.

- Charter Party Form:	EMGL form, all terms to be agreed.  The contract will consist of an enabling-type EMGL Master Time Charter Agreement, with individual Vessel Orders for each Vessel.

Operating Requirements

Below are the EMGL operating requirements that must be met. There are 2 examples offshore terminal tug designs that could be used by EMGL as base spec - Robert Allan Rampage 6000 or Damen 6015.  Details of such tug designs may be requested by interested suppliers EM.Guyana.Procurement@exxonmobil.com
Note: These tug designs are examples only, and suppliers may propose alternative tug designs.

New Tug Build Requirements

We expect the new build tugs to have all the operating requirements as follows

- Design Draft: Capable of transits within the Demerara River up to GYSBI shore base at design draft conditions, including the minimum UKC.

- Vessel Size – in the region of 50 to 60m length.

- Capable of remaining fully operational offshore for 60 days continuous without taking stores, fuel or water requiring a port call.

- Service speed of between 12 and 15 knots

- Modern 360-degree bridge layout including ECDIS capable of single man operations but space for maintaining a 2-man DP bridge watch.

- Re-enforced / shatterproof fwd bridge windows for bow towing operational safety.

- DP2 Capable

- Bollard Pull: Minimum of: 120T Ahead and Astern pull

- FiFI 1 minimum capabilities

- 300T Hydraulic Anchor Handling Towing Winch: Capable of holding 1200m x 64mm wire – fitted with tension Meters, spooler and Towing line deployment lengths

- Towing winch with capacity to spool towing wire, stretcher and pennant.

- Hydraulic Storage reel capable of holding 1 off - 1200m x 64mm wire.

- 2 x 15T (min) Tugger Winches Port and Starboard located on the fwd main deck.

- 2 x 10T (min) Capstans located at the stern of the vessel Port and Starboard side.

- 2 x 10T vertical stern rollers.

- Minimum SWL 150 MT stern roller

- Minimum SWL 300 MT Hydraulic Remote Control Retractable Aft Towing pins in way of the C/L

- Minimum SWL 300 MT Hydraulic Remote-Control Aft Shark Jaws in way of the C/L

- Gob eye / Deadman at suitable location for stern towing arrangements.

- Rollers and fairleads of suitable SWL fitted down the deck length at various locations to allow for tugger winches and capstans to be led to the main deck at various pulling angles.

- Pilot boarding platforms suitable for transferring personnel to and from the tug by means of pilot ladders deployed from crude oil tankers. Platforms may be of a removable design.

- 60m3 Minimum Oily / Water residue tank / sump for draining floating hoses on deck during hose maintenance work.

- Oil recovery Class notation required.

- Wide side gate located on starboard side for 20” floating hose maintenance.

- Hose guide / fairlead with rollers to allow for smooth floating hose deployment and direction through the side gate.

- Fwd Winch or Aft Winch to Forward Tunnel capable of holding 800m of properly rated diameter Dyneema line not less than 300 MT MBL based on 120MT bollard pull. System must be fitted with spooler guide, tension meters and Towing line deployment length display to allow for Dyneema (Soft) Tow line to be used and deployed from the bow.

- Stainless steel bow staple for deployment of soft towing arrangements.

- Clear Deck Area – 250 to 300m2 approx. Combination of wooden and steel deck

- 1 x FRC suitable for deployment in 3.0m sea conditions.

- Full CCTV Camera System on deck, E/R and winch area.

- FUELTRAX system fitted on all consumers.

- Automatic Tank Gauging (ATG) system + capabilities for manual soundings of fuel / water ballast and other liquid tank.

- Power plant (engines and generators) capable of being run on biofuels / renewable fuel sources and compliant with all emissions regulations and requirements. Fuel usage and efficiency should be considered at all stages of the vessel design.

- Fresh Water Production system suitable of maintaining water levels for full complement onboard.

- 20 MT Knuckle Boom Crane for stores and hose maintenance situated on the main deck on same side has hose gate.

- Bow fendering is suitable for tug pushing operations with crude oil tankers and personnel transfers from the bow.

- Stern fendering at each quarter suitable to allow for pressing alongside crude oil tankers and conducting personnel transfer from the stern.

- Side fendering adequate and of robust design to allow for pressing alongside crude oil tankers.

- Vessel at berth - mooring plan / design with fairleads and bitts suitable for a 4m rise and fall in tide. Mooring lines will be lead through fairleads and secured on rated mooring bitts.

- Fueling: 4” Todomatic connection both on Port and Starboard side,

- Minimum Accommodation for POB 20 person’s total – minimum additional of 1 x client’s cabin and 2 x 2-man rooms in addition to vessels compliment.

- Mess Room Adequately sized for vessels normal compliment

- Galley, fridges and storage suitable for 60 days continuous work offshore.

- Laundry Adequately sized for vessel normal compliment

- Adequate storage compartment for storing chemicals / ship stores.

Below is EMGL’s preferred experience requirements for Operators of Offshore Terminal Tugs supporting operations in Guyana.

- Vessel Technical Operator to have minimum of 5 years’ experience of Offshore Tug operations, of tugs of minimum 100t Bollard Pull.

- Vessel Technical Operator to have experience in operating Dynamically Positioned (minimum DP2) vessels.

- Vessel Technical Operator to have experience of Offshore Tug operations, specifically Tanker & FPSO Tandem Mooring Operations.

- Vessel Technical Operator to be registered on OCIMF database, and have submitted Offshore Vessel Management Self-Assessment (OVMSA).

- Vessel Technical Operator to utilize and apply Offshore Vessel Inspection Questionnaire (OVIQ) to vessels.

- Scope of Services

ExxonMobil Guyana Limited (EMGL) plans to put in place a charter party for provision of New Build Offshore Terminal Tugs, primarily to assist and control export tankers (ET) of up to 320,000 metric tons deadweight (e.g. VLCC size) during mooring, unmooring and station keeping for cargo loading operations while moored in tandem to a spread-moored, non-weathervaning FPSO operations.

The service agreement will be accomplished via one of the following or a combination of

- Tender utilizing a charterer pre-negotiated ship building contract for new builds with multiple shipyards and or

- Tender from shipowner proposing new build Offshore Terminal Tugs that meets requested specification.

The scope of this Request for Information (RFI) is to identify suppliers who can provide New Build Offshore Terminal Tugs to EMGL as follows:

- Trail the light draft Export Tanker (ET) while tethered to the ET’s stern at speeds up to 5 knots on approach to the FPSO.

- Provide heading, speed of approach and stopping control of the ET as it maneuvers into loading position off the stern of the FPSO.

- Transfer the marine floating cargo hose(s) by means of towing from the FPSO to the ET and position under the ET’s midships manifold crane for hose pick-up.

- Provide static tow heading and position control to the ET as required during loading.  It is expected that one Vessel will be connected to the ET’s stern throughout the loading evolution to provide holdback and steering forces, with two other tugs connected to the ET’s bow and quarter to provide corrective heading forces.  Coordination amongst the tugs will be provided by the Charterer’s Mooring Master or his designate located aboard the ET. The new build tugs shall be designed to perform towing operations on ET bow, quarter and main pullback positions.

- Be connected to the ET stern on departure from the FPSO to provide pullback, steering and stopping control.

The secondary missions of the Vessels shall include, but not be limited to, the following:

- Maintenance of the marine floating hose string and hawsers

- Oil spill containment boom deployment

- Dispersant spraying

- Field standby and rescue

- Personnel transport and transfer from FPSO to ET

- Other support requirements to be described in the applicable work order

- Oil Spill Containment Boom Deployment

      - Deployment, positioning and recovery of oil spill containment boom provided by Charterer

- Containment boom will be stored on, deployed from and recovered to Charterer-supplied reels with associated power units.

- Reels will be loaded from pre-positioned offshore facilities and onshore base locations.

- The Vessel will work alone or in conjunction with other vessels to maneuver and control the deployed boom at speeds no more than one (1) knot, for sustained periods and in consideration of the prevailing weather and currents.

- Dispersant Spraying

- Installation and use of portable spray application systems provided by Charterer.

- Mobilization and on-deck securing portable dispersant tanks pre-positioned at onshore bases and offshore facilities.

- Field Standby and Rescue

- The Vessels will be required to provide standby, off-ship firefighting, and rescue operations including the launching and recovery of rescue boats and deployment and use of rescue nets.

- The Vessel will meet Classification Society requirements for a “FiFi-1” notation, including the ability to fight off-ship fires and to approach burning structures to remove personnel.

- Personnel Transport and Transfer

- The Vessel will provide the transport and transfer of personnel primarily between FPSO and ET or otherwise dictated by the Charterer.

- Local Content Requirements

3.1 	EMGL’s local content strategy is important to our business. Its elements are integrated into our daily processes and procedures and guide the way we work today and plan for tomorrow.

3.2 	Using a multi-tiered approach, EMGL focuses on building workforce and supplier capabilities in conjunction with strategic investments in the local community.

3.3	During the bid process, all prospective EMGL contractors are required to demonstrate awareness of the Local Content Act (2021) and its potential application to their work under the scope.

3.4 	Bidders may be expected to include in their submission a preliminary local content strategy outlining how they will deliver on the local content expectations outlined below and those captured in the Act, and, upon award, implement an acceptable plan.   Should the successful bidder be required to submit local content plans to the Government of Guyana under the Local Content Act (2021) they will also provide a copy of those plans to EMGL.

3.4 	The following requirements apply to orders that will be performed, or with exercise of contractor’s best efforts could be performed, wholly or in any part in Guyana (including without limitation the Guyana Exclusive Economic Zone) or with utilization of any Guyanese subcontractor, vendor, or similar Guyanese content.

3.5 	The below requirements specific to EMGL contractors are in addition to the Local Content Act (2021) issued by the Government of the Cooperative Republic of Guyana; in the event of a conflict between these requirements and those of the Government of Guyana explicit instruction must be sought by EMGL.

REQUIRED ACTIVITIES:

A.	Contractor Procurement Activities:

To support EMGL’s objectives of giving preference to qualified local suppliers from Guyana, a contractor shall:

        - Give fair and adequate opportunity and first consideration to the following:

- The purchase of Guyanese goods and services provided such goods and services are available on a timely basis of the quality and in the quantity required by the contractor at competitive prices.

- Utilization of Guyanese contractors insofar as they are commercially competitive and satisfy the contractor's financial and technical requirements and meet the requirements of this section.

        - Establish appropriate tender procedures for the acquisition of goods, materials, and services, which shall ensure that Guyanese contractors are given fair and adequate opportunity to compete for the supply of goods and services.

        - Establish procedures to provide the contractor’s Guyanese subcontractors feedback following contracting activities.

        - Ensure that the contractor’s subcontractors comply with the requirements contained in this section.

        - Steward the contractor’s, and its sub-contractors’, service performance, including local content development progress in accordance with these requirements and the contractor's local content plan.

B.	Contractor’s Employment Opportunities

To support EMGL’s objectives of giving preference to qualified local personnel and suppliers from Guyana, a contractor shall:

        - Prepare and implement a program to give fair and adequate opportunity and first consideration for employment of Guyanese nationals having appropriate qualifications and necessary experience to perform job responsibilities commensurate with the contractor’s scope of work.

        - Ensure that the contractor’s subcontractors comply with the requirements contained in this Section.

        - Steward the contractor’s and its sub-contractors’ local content development progress in accordance with this guidance and the contractor's local content plan.

C.	Contractor’s Use of External Resources:

1.	Centre for Local Business Development

- To facilitate transparent local hiring and contracting, a contractor is encouraged to consider using the following capabilities provided by the Centre for Local Business Development (CLBD):

          - Advertising upcoming vacancies and tenders with potential local content.

          - Publishing Vacancies/Expression of Interest/Tenders in the public domain for approximately two weeks to one month (as relevant) via the Centre for Local Business Development Hub and CLBD Facebook/Website.

          - Utilize the Hub as a resource market, for sector insights and for market intelligence related to Guyanese contractors that meet requirements.

          - Plan communications for announcements of tender award/hiring/training as appropriate.

          - Provide opportunities for feedback following Expression of Interests/Request for Information (RFI) and bids to Guyanese vendors.

- A contractor should also consider engaging, and actively collaborating with, the Centre for Local Business Development.

(i)	The Centre’s role is to facilitate communication between suppliers, contractors and subcontractors, with a goal to build the capacity of Guyanese small and medium enterprises. The Centre will maintain a Supplier Registration Portal and is able to provide a mechanism for advertising opportunities for Guyanese goods and services, for a contractor’s use.

(ii)	EMGL does not endorse or recommend any supplier listed on the Centre Hub.  As such, a contractor should perform appropriate due diligence on any supplier prior to use. The Centre is located in Georgetown at 253-254 South Road, Bourda on the top floor of the IPED Building. The website is: www.centreguyana.com

2.	Online & Social Media – Publishing Vacancies/Expression of Interest/Request for Information in the public domain for approximately two weeks to one month, as relevant, via relevant online and social media sites (EMGL, Local Content Secretariat, contractor, and newspapers).

D.	Contractor’s Local Content Metrics Reporting (If Required)

      - A contractor shall provide local content reporting metrics using an EMGL-provided format or digital tool.

      - Metric reporting requests will be sent at least twice annually to the contractor.  EMGL reserves the right to change the frequency of metric requests to the contractor.

      - Should the contractor be required to submit metrics to the Government of Guyana under the Local Content Act (2021), EMGL may request a copy of those reports.

- Safety, Security, Health & Environment (SSHE)

ExxonMobil Guyana Ltd and its partners are committed to providing a safe, healthy, and environmentally sound workplace for employees, contractors and subcontractors. Interested Suppliers must have a comprehensive Health, Safety, Environmental and Quality management system that meets, at a minimum, all requirements of ExxonMobil, and local regulatory processes

- RFI Submission Instructions

Please complete Section 7 - Questionnaire, elaborating on each item as appropriate and keeping the following in mind:

- Please answer the questions as completely as possible and advise if you cannot answer a particular question.

- Please note the word limit for each section. Submitters should not exceed the word limit.

- Please submit your response in electronic form using any of the following electronic formats: Microsoft Word, Microsoft Excel, Adobe PDF, etc.

- Please do not include any links to documents or sites in your responses as proof points for qualifications. EMGL cannot access external links.

- Your e-mailed responses should be limited to a reasonable size (<5MB). If the response is >5MB, then multi-part email submissions <5MB are acceptable.

- Please do not submit pricing or price proposal information at this stage.

- Your submittal should not include information which you consider proprietary, trade secrets or confidential.

- Please submit all questions to email address:  EM.Guyana.Procurement@exxonmobil.com

- Key Dates in the RFI Process

September 17, 2026	RFI to be issued

October 1, 2026	Planned RFI clarification session to be held via Zoom

To register for this session:

Registration Form - ExxonMobil Clarification Forum on Upcoming            RFI's

12:00 noon -1: 00 p.m. GYT (11:00-12:00p.m. CST)

Please submit all questions to:

EM.Guyana.Procurement@exxonmobil.com

October 19, 2026	Deadline for supplier to submit completed RFI (firm)

Q4, 2026	Evaluation of RFI responses (estimated)

Q4, 2026	RFP invitations (estimated)

- Submission of RFI with all complete documents

All documents required to be submitted are due no later than 5:00 PM, October 19, 2026.

Contact for questions: EM.Guyana.Procurement@exxonmobil.com

How to Submit:

1. Complete Required Documents and include in an email

- Completed Questionnaire (Section 8 attached excel)

- Completed Prospective Business Associates Questionnaire (PBAQ)

- Copy of the company’s Articles of Incorporation (including the page with the Directors/Management and details of nationality)

- Copy of the company’s Certificate of Registration (company TIN) from the Guyana Revenue Authority (or equivalent if international)

- Copy of company’s organizational chart with details on nationality of all personnel

2. Send Email with the following title format:

Supplier Name – RFI 15-2026 Offshore Terminal Tug – Final RFI Submission

3. Send email to: EM.Guyana.Procurement@exxonmobil.com

4. Kindly note it’s recommended but not a requirement to register at the CLBD in order to respond to any of our RFI’s as this request is open to all interested companies.